Phantom.Home.doc

05/17/2000

GNU Phantom.Home

Copyright (C) 1999, 2000 by Joe Thielen <jthielen@accesstoledo.com>

-------------------------------------------

===============================

TABLE OF CONTENTS:

1.  Introduction/Philosophy

2.  GNU Phantom.Home Installation

3.  Phantom.Homed Basics

4.  Phantom.Homed Advanced

5.  Phantom.Home.Console Basics

6.  Phantom.Devices

7.  Phantom.Groups

8.  Autolighting

9.  Hardware/Electrical Info

10.  Thanks/Legal Stuff

11.  Comments/Questions/Complaints/Contacting Me...

===============================

1.  Introduction/Philosophy

This program was a spin-off of my Phantom.Security software which, in-turn, was

a spin-off of an attempt at creating a Linux controlled robot.  I figured, if

I can read input from devices into the computer, why can't I control devices

as well?!?  So, I jury-wired my basement, constructed a very simple electronics

circuit which can control a 12V DC relay and then built it.  Now I control all

the lights (3 sets of them) and even a vent fan from my Linux machine!  I've

looked on the Internet, but about the only thing I've found comparable to this

is the X10 software, but, of course, that's not free nor open-sourced...

2.  GNU Phantom.Home Installation

There are 4 steps to completing a simple GNU Phantom.Home setup.  These are:

1.  Make the Phantom.Home.Controller.

2.  Make the Phantom.Home.Modules.

3.  Plug everything in.

4.  Configure the software.

Step 1.  Make the Phantom.Home.Controller.

Included in this package is a JPEG image of the circuit diagram.  It's

relatively simple, and even those of you with limited electronics assembly

should be able to make the Phantom.Home.Controller.  If your an intermediate

or advanced electronics hobbiest (or expert) and want to do something else

with the circuit (like modify it to control something else) then go right on

ahead!!!  The parts for building the controller are readily available at a

large Radio Shack near you (only the larger ones carry more electronics

components anymore).  The parts should be rather cheap.  You should be able

to build Phantom.Home.Controller for less than $25 (not counting the relays for

the Modules).

Step 2.  Make the Phantom.Home.Modules.

This part isn't necessary, as you can place the relays on the Controller itself,

but I'd rather have 12VDC running around than 120VAC.  The Modules are simply

12VDC relays mounted in a box.  You have a 120VAC cord with a standard plug

coming out of one end, and out of the other, you have either a wall jack

or a jack on a cord.  Somewhere on the box you have your 12VDC controlling

wire running into it, too.  All of these cords run into the relay.  Now, you

can hard-wire a relay, but it would be much easier to get the relay mounting

piece, and hard-wire that.  That way, if your relay burns up, then you can

simply un-plug it and plug in a new one without soldering new connections.

These modules can be placed right between the appliance you want to control

(i.e. a desk lamp) and an electrical source (i.e. a wall jack).  Of course,

you can control your main lights with this system too, it just takes a little

tinkering with your houses wiring.  PLEASE EXERCISE EXTREME CAUTION WITH

120VAC WIRING!  MAKE SURE THE CIRCUIT BREAKER IS 'OFF' FOR THE WIRES YOU

INTEND TO WORK WITH.  WORKING WITH LIVE 120VAC WIRING CAN BE HAZARDOUS TO YOUR

HEALTH AND MAY EVEN BE FATAL!  OK?

Step 3.  Plug everything in.

After you've constructed the Phantom.Home.Controller, and you

Phantom.Home.Modules, you're almost ready to go!  First, plug your Controller

into the computer and a power source.  Second, run the wiring from your

Controller to your Modules.  Third, plug your controlled devices into your

Modules.  Fourth, plug the Modules into an electrical source!  That's it!

Step 4.  Configure the software.

Now you've got all this neat stuff in place, what do you want to do with it?

Make it work, of course!  Updated compiling instructions are located in the

Phantom.Home.README file, but it's usually done my running 'configure', then

'make', then 'make install'.  This will copy the binaries and docs to the

default directory of /home/Phantom/home.  This can be changed by modifying

the top-level configure.in (only for those familiar with this!).  Then edit the

Phantom.Devices & Phantom.Groups files (instructions for the last two can

be found in sections 6 & 7).

Now you have everything you need for the Phantom.Homed program itself.  But

Phantom.Homed is simply an interface between your computer and the

Phantom.Home.Controller itself.  The way to manipulate the devices on the

Controller is via some sort of interface to Phantom.Homed.  Included in this

dist. is the text-based interface: Phantom.Home.Console.  A VRML (Virtual

Reality Modeling Language) web based interface is also available, but can not

be included in this distribution because of GNU conformance rules.   There is

currently no free or GPL'd VRML browser out there.  So someone please write one

so I can include Phantom.Home.VRML in this package! :)

Phantom.Home.Console is a text-based command interpreter.  You give it commands

like "turn off light1", and the device you specify as 'light1' in the

Phantom.Devices file will turn off.  "turn on lights" will turn off the devices

you specify for the group 'lights' in Phantom.Groups.

NOTE:  You can use multiple interfaces AT THE SAME TIME if you wish to.

Phantom.Homed was designed to accommodate multiple programs.  You can

even have multiple copies of Phantom.Home.Console running at the same

time as Phantom.Home.VRML!

3.  Phantom.Homed Basics

The Phantom.Homed program can be used as a standard program, or you can put it

in the background (just like a daemon).  To run the program, simply run the

executable (i.e. just type Phantom.Homed at the command line and press <ENTER>).

Obviously, Phantom.Homed must be either in the current directory or in a

directory in your $PATH.  To run the program as a daemon, type in:

Phantom.Homed & <ENTER>

And the program will run in the background.

There are two command-line arguments to Phantom.Homed, -l and -v.  Argument

-v is for 'verbose'.  When invoked, it will send all status information to

stdout (usually the screen).  Usually you don't want to invoke the -v argument

when the program is running in the background, for it makes for a messy session!

Argument -l is for 'logging'.  As opposed to the -v option, -l will send all

status information to a file named Phantom.Log in the home directory

So, to run Phantom.Homed as a daemon in the background using logging, you'd do:

Phantom.Homed -l & <ENTER>.

And off it goes.  When you run the program, it will create a file called

Phantom.PID.  This file contains the Process ID (PID) of the Phantom.Homed

program currently running.  So to stop the Phantom.Homed program, you'd type:

kill -9 PID

Where PID is the number from the PID file.  A nicer way to shut down the

daemon is to feed it the number '9' via it's API.  To do this you must be in

the homedir.  Execute:

echo 9 > Phantom.Home.Out

And the program will shut down in one or two seconds.  It will automatically

clean up after itself using this 'nicer' method.  It will delete the PID file

as well as write an END line to the log file (if applicable).

4.  Phantom.Homed Advanced

This section is for users wanting to develop a custom interface to

Phantom.Homed!  If you don't plan on doing this, skip this section!

Phantom.Homed uses a file-based API (Application Program Interface, or

something :) ).  This means that any number of programs can use Phantom.Homed.

You can even create a custom interface for Phantom.Homed!!!  The API is file-

based, meaning that you simply create a file with the command (or request)

for the Phantom.Homed program.  The name of the file is Phantom.Home.Out.  It

must be created in the homedir.  The content of the file is a single character.

Character	Meaning

0 		Tells Phantom.Homed to return the current status of the

devices in their binary form (0=off, 1=on).  i.e. 01011010

It puts this in the file Phantom.Home.In.

1-8		Switch the current state of device x (1 through 8).

9		Stop execution & end program.

h		Switch all devices in group 1 on.

i		Switch all devices in group 2 on.

j		Switch all devices in group 3 on.

k		Switch all devices in group 4 on.

l		Switch all devices in group 1 off.

m		Switch all devices in group 2 off.

n		Switch all devices in group 3 off.

o		Switch all devices in group 4 off.

Phantom.Homed only reads the first character of Phantom.Homed and ignores any

character not indicated in the above list.  So if you precede your command

with a space (i.e. ' 1') it will not work.  However, if you put a space behind

it (i.e. '1 '), it will work, as long as the character is the first in the

file.  Phantom.Homed deletes Phantom.Home.Out as soon as it's finished reading

it, so the same command will not be executed several times in a row.  If your

program makes use of the '0' (status) command, make sure it deletes the

Phantom.Home.In file as soon as it's done reading it in case another program

needs to use it.

API NOTE:  As seen by the above, 1-8 just switch the state of the device,

they don't specify a specific on or off state.  So it would be wise to precede

your commands with a status request first.  To illustrate let's assume device

one is a table lamp, and it's currently in the 'off' (0) state.  If you send

a '1' command via the API with the intention of turning the lamp off, you're

going to end up turning it on.  So it would be wise to first send a '0'

request via the API to establish the status of the lamp.  After reading

Phantom.Home.In (returned from Phantom.Homed as a result of your '0' request),

you can then verify that the lamp is already off.  This reasoning behind this

is that more than one program may be using Phantom.Homed at the same time,

so even if your program is keeping track of device status from it's own

commands, it may be wrong if another program is using Phantom.Homed too!!!

5.  Phantom.Home.Console Basics

When you first start the GNU Phantom.Home software it displays the GPL (GNU

Public License) along with messages stating that it's read the Phantom.Devices

and Phantom.Groups files and the current version.  It will then bring you to

a prompt.  This prompt is where all of the human-computer interaction takes

place.  Using this prompt, you give GNU Phantom.Home commands like 'Turn off X',

where 'X' is on your devices named in either the Phantom.Devices or Phantom.

Groups files.  X could be a simple desk lamp (i.e. light or light1), or it

could be your jacuzzi!  For a complete list of commands accepted by

Phantom.Home, see the Phantom.README file.  Each new version will usually

add new commands, so I'll leave that in the Phantom.README instead of in this

document.

In Phantom.Home you have Devices as well as Groups of Devices.  For instance,

let's say you have three lights (as I do).  You have a single overhead light

and two table lamps.  You have individual control over these items by typing

in commands like 'Turn on light1' or 'Turn off light3'.  But you want to be

able to control all three with one command, also.  No problem!  Just define

a group containing these devices in your Phantom.Groups file.  For the sake

of this discussion, let's call the group 'lights'.  So, you can now type in

a command like 'turn off lights', and viola!  All three of the devices

are switched off (even if only one or two of them is on).  You can then

say 'turn on lights', and all three lights come on.  Ahh... the bliss of

laziness.

To exit the program, just use the 'exit' or 'quit' command.  You'll get a

'Thank you for using Phantom.Home.Console version xxxx' message when you leave.

6.  Phantom.Devices

The format of an entry in the Phantom.Devices file is this:

DEVICEx:Enabled/Disabled:Aliases

Here is an example:

DEVICE1:Enabled:light1,mainlight

This defines DEVICE number 1 (DEVICE1).  It is enabled (so the software can

control it), and it has two aliases: 'light1' and 'mainlight'.  That means

you can issue commands like 'Turn on light1' or 'Turn off mainlight', and

control the same device using different aliases.  Aliases should be a group

of words separated by commas (,).

You can have up to eight (8) devices in the Phantom.Devices file.

7.  Phantom.Devices

The format of an entry in the Phantom.Groups file is this:

GROUPx:Enabled/Disabled:Aliases:Devices

Here is an example:

GROUP1:Enabled:lights,allights:1,2,3

This defines GROUP number 1 (GROUP1).  It is called 'lights' or 'alllignts',

and contains the devices 1, 2, and 3.  This means when you type in a command

like 'Turn on alllights', it will turn devices 1, 2, and 3 on.  Likewise, a

'turn off lights' will turn off all three devices (regardless of whether they

are already off or not).

Of course, you can assign a group to a singular device also:

GROUP2:Enabled:tablelamp:2

All Aliases and Devices should be lists separated by commas.

You can have up to four (4) devices in the Phantom.Groups file.

8.  Autolighting

As of Phantom.Home Beta0.62, autolighting is now supported!  Autolighting

is a way of automatically controlling your lighting via a door magnet on

the door leading into a room (this can be modified for multiple doors, or

for using a motion sensor).

To use the autolighting features, you must first connect one end of a normally

open magnetic switch to the parallel ports ground, and the other end to pin 11.

Then you have to start Phantom.Homed with the '-a' option to enable

autolighting.  When you're ready to leave the room & enable autolighting,

type in 'im leaving' (in Phantom.Home.Console).  NOTE:  You still have to shut

off the lights off via Phantom.  The next time you enter the room, whatever

device you have connected to D4 (i.e. Device 4) will turn on!  To leave

the light (or whatever is connected to D4) on, type in 'im here' or 'im back'.

If you don't type in 'im here' or 'im back', the light will turn off in 3

minutes (180 seconds).  The device and time length can be modified in the

Phantom.Homed.cpp source (in the Door_Check function).

Enjoy!

9.  Hardware/Electrical Info

The circuit diagram is relatively simple (as is intended).  However, if you

have a device that is more complicated than ON or OFF, then you can modify

the hardware/software to your content.  If you have a jacuzzi and want to

control the temperature from your computer, you can go right ahead, as long

as you know how :).

Please be EXTREMELY CAREFUL when dealing with electricity.  ESPECIALLY 120VAC

house wires.  NEVER DEAL WITH LIVE WIRES!  Flip the circuit breaker to 'OFF'

before you try to deal with house lines.  Please be careful!

As for the wires running between the Controller and Modules, you can manage

this in one of several ways.  If your sure your setup isn't going to change

then you can hardwire the 12VDC line from the Controller to the Modules.

However, a better approach would be to use RCA jacks and plugs.  You can

buy a board from Radio Shack that has 8 female jacks on one board and mount

that to your Controller.  Then, put a single female jack on each of your

Modules.  Now you can use either pre-made OR home-made RCA cables to run from

your Controller to your Modules.  Obviously the RCA cables should have male

plugs on BOTH ends.

10.  Thanks/Legal Stuff

I'd like to thank Riku Saikkonen <Riku.Saikkonen@hut.fi> for his mini-HOWTO on

IO-Port-Programming.  I used his example code almost verbatim in my early

Beta versions.  This is the only document I could find that smacked me in the

face and said "Look idiot, it's that easy!".  And it worked.  Thanks!

I'd also like to thank Scott Roberts for his help with the electronics

components and circuit diagram creation.  He was a big help in creating the

circuit.  Thanks Scott!!!

I'd like to thank Rick Crelia <rcrelia@mail.RATH.PeachNet.EDU> for his input

in making GNU Phantom.Home a better package!   He evaluated Phantom.Home for

the GNU and was very helpful in making it a GNU program.  Thanks Rick!

This software comes "AS-IS" with NO WARRANTIES expressed or implied.  This

software is free for all to use, copy, edit, compile, distribute, and hack.

I only ask that credit be given where credit is due, and that if a modified

version of this software is distributed, that a copy of the original package

be included.  However, it is free software and open sourced so mod mod mod!

However, I am not responsible for any damage the software or circuit diagram

may cause.  Nor am I responsible if the circuit is built wrong and starts

something on fire.  Nor am I responsible if you spill Jolt cola into the

power transformer and start your house on fire.  So please exercise caution

when assembling and using electronic components.  Please be safe!

This software is copyrighted, and I've placed it under the GNU Public License.

Please support the GNU and the free software community at http://www.gnu.org!!!

See the file COPYING for details...  here is the distribution statement:

This program is free software; you can redistribute it and/or modify it

under the terms of the GNU General Public License as published by the

Free Software Foundation; either Version 2 of the License, or (at your

option) any later version.

11.  Comments/Questions/Complaints/Contacting Me...

I'd love to hear anything you have to say about the software, so please drop

me an e-mail at jthielen@accesstoledo.com or visit my website at

http://www.joethielen.com!

================================================================================